Terms of Reference (ToR)

Supply, Delivery, Installation, and Commissioning of Solar Power System for DRC Sudan Offices

1. Background

The Danish Refugee Council (DRC) Sudan aims to reduce dependency on fuel-powered generators and unreliable grid electricity by investing in a sustainable solar power system. The system will provide reliable and environmentally friendly energy to DRC Sudan offices, ensuring cost-efficiency and continuity of operations.

2. Objective

The objective of this assignment is to contract a qualified supplier to supply, deliver, install, and commission a complete solar power system, while also providing training, after-sales service, and long-term maintenance support.

3. Scope of Work

3.1 Supply and Delivery

Solar Panels (130 modules or similar)

- Type: Photovoltaic Solar Panels, A-Grade quality

- Power Output: 580-590 W

- Current: 14.8A

- Voltage: 48V

- Warranty: 25 years

- Must include barcodes/serial numbers and manufacturer certificates to verify authenticity and quality.

Mounting Frame

- Robust 18-gauge metallic frame, durable and suitable for supporting solar panels

- Compliance with international safety and quality standards

Lithium Batteries (4 units)

- Capacity: 10kW, 200A, 48V

- Cycle Life: 6000 - 8000 cycles (or equivalent)

- Warranty: 10 years (supplier must clearly state whether warranty covers replacement, repair, or both)

- Must include barcodes/serial numbers and manufacturer certificates for validation.

Inverters (5 units)

- Type: Hybrid, 3-phase, on-grid inverter

- Capacity: 10 – 14 KW

- Conversion: DC to AC, AC to DC, AC to AC, DC to DC

- Connectivity: Internet/Wi-Fi/GPRS capability for:

  - Online monitoring of energy production and consumption

  - Remote fault detection and diagnostics

  - Cloud-based data logging and reporting

- Warranty: 5 years

- Must include barcodes/serial numbers and manufacturer certificates.

Electrical Components and Accessories

- Cables: 6mm, high-quality, fixable

- Fuse Boxes: AC and DC fuses, overload protection, pass bar, and accessories

- PVC ducts for safe and durable cable routing

- Additional accessories: connectors, mounting kits, and other required items for a complete installation

Transportation

- Delivery of all items to DRC Sudan offices (inclusive of transportation cost).

3.2 Installation and Commissioning

- Complete installation of solar panels, mounting frames, batteries, inverters, fuse boxes, and accessories.

- Testing and commissioning of the system to ensure operational readiness.

- Ensure internet connectivity for inverter(s) and configure online monitoring platform for DRC Sudan access.

- Compliance with international safety and electrical standards.

3.3 Training and Handover

- On-site training for DRC staff covering:

  - Daily operation and monitoring (including online system usage)

  - Cleaning, safety, and preventive maintenance

  - Troubleshooting of basic issues

- Provision of manuals, warranty documents, barcodes/serial numbers, and maintenance guidelines.

3.4 After-Sales Service

- Minimum 5 years of after-sales service including:

  - Preventive maintenance and scheduled checks

  - Error fixing and troubleshooting (onsite and remote)

  - Replacement/repair of faulty components covered by warranty

  - Guaranteed availability of spare parts during the service period

4. Supplier Responsibilities

- Supply original, new, A-Grade certified equipment (no refurbished items).

- Provide barcode/serial numbers and manufacturer certificates for validation of panels, inverters, and batteries.

- Ensure proper installation and system functionality.

- Ensure internet connectivity for inverter(s) and demonstrate online monitoring features to DRC staff.

- Conduct cleanup of the worksite after installation.

- Provide all warranty documents and clarify scope (repair vs replacement).

- Deliver after-sales service and maintenance for at least 5 years.

5. Supplier Selection Criteria

- Legally registered company in Sudan (or internationally recognized) with valid licenses.

- Proven experience in designing and installing solar systems of at least 50KW (minimum 2 similar projects; references required).

- Technical team including at least 2 certified solar engineers with:

  - Minimum 3 years’ experience in solar system installation

  - Relevant qualifications/certifications (copies required)

- Proven ability to supply A-Grade solar panels with barcodes/serial numbers and manufacturer validation.

- Demonstrated ability to provide training and long-term support.

- Proven experience providing internet-connected inverters with online monitoring platforms.

- Strong warranty commitments with clear definitions (replacement/repair).

- Competitive and transparent financial proposal (covering equipment, transportation, installation, and service).

- Ability to complete delivery and installation within the agreed timeframe.

6. Deliverables

- Fully installed and commissioned solar power system.

- Configured online monitoring platform with access credentials for DRC Sudan.

- Training session conducted for DRC staff.

- Handover package including manuals, warranty documents, barcodes/serial numbers, and system diagrams.

- Signed 5-year after-sales service agreement.

7. Duration

Supply, delivery, installation, and commissioning should be completed within [insert timeframe, e.g., 8 weeks] from contract signing.

8. Payment Terms

Payment will be made in tranches linked to milestones:

- Upon delivery of equipment (with barcode/serial verification)

- Upon completion of installation and commissioning (including online monitoring setup)

- Upon final acceptance by DRC Sudan
Retention may apply until successful commissioning and handover.