Term of references

Supply of materials and construction of truss structure measuring 7 meters in width and 15 meters in length high 3.5m.

Side wall height: 3.5 meters

Roof slope: approximately 25%

Roofing: 35-gauge zinc sheets with thermal insulation (Totaia) installed above the roof between the zinc sheets and rafters

Roof support: Z-purlins fabricated from (4 × 8) steel pipes

Structural System

Columns fabricated from 14 cm I-sections, spaced every 5 meters

X-bracing provided in alternate bays, including first and last bays

All steel members connected using high-tensile bolts

Steel surfaces coated with two layers of anti-corrosion paint and finished with PA-approved color

Foundation and Substructure

Isolated foot 0.8*o.8 depth 0.8m (0.6 under ground 0.2 above ground level)

Floor: Ceramic tiles after 15 cm well compacting backfilling

Wall base: red brick wall, finished with plastering and painting

Doors

One main sliding door 3 m high 3 m

Doors fitted with industrial-grade hinges and locks

All materials shall be new, and construction shall be carried out by qualified personnel in compliance with international quality, safety, and engineering standards. The truss shall be delivered fully functional and ready for immediate use, in accordance with the Site Engineer’s instructions