| Plan International Sudan White Nile Program Area Tender No. (WN 01 FY2027) INVITATION TO TENDER Organization: Plan International Sudan – White Nile Program, Kosti Office Project: Construction of 2 Classrooms (9m × 5m) in Altonsa Community, Al-Dawaim Locality, Southern Rural. Overview: Plan International Sudan invites technical and financial proposals from qualified contractors and companies for the construction of two classrooms as detailed in the tender specifications and conditions below. Key Dates and Procedures: - Tender Documents Distribution: Starting Sunday, 20 September 2026 - Distribution Location: Plan International Sudan Office, Kosti, White Nile State (during official working hours) - Deadline for Document Collection: Monday, 28 September 2026 - Bid Submission Deadline: Thursday, 1 October 2026 at 10:00 AM - Submission Location: [Specify office/location] ## BID SUBMISSION CONDITIONS AND REQUIREMENTS ### General Bidding Conditions - Currency: All financial offers must be submitted in Sudanese Pounds (SDG). - Price Validity: Quoted prices must remain valid for a minimum of one (1) month from the bid submission deadline. - Bid Completion: Bidders must carefully review all tender conditions and specifications prior to submission. Any modifications to bid documents that are not signed and stamped by the bidder will result in automatic disqualification. Unsigned or unsealed amendments will be excluded from evaluation. - Committee Authority: The Bid Evaluation Committee reserves the right to reject any bid or cancel the tender process at its discretion for technical or administrative reasons deemed necessary. - Winner Orientation: The successful bidder must meet with the Evaluation Committee for orientation and coordination with the technical authority. The awardee must agree to deliver services in full compliance with all specified conditions. - Bid Submission Method: Bids must be submitted in sealed envelopes closed with red wax and placed in the designated bid box at Plan International Sudan's office in Kosti (Sarayat neighborhood, east of the Officers Club). - Bid Form Completion: All bidders must complete the attached bid template to ensure uniformity across submissions. The form must be signed and stamped. Bids without a completed bid notification form will be disqualified. - Tax Documentation: Clearly state the Value Added Tax (VAT) amount and attach: - Assignment letter for VAT collection - Copy of tax registration certificate - Acknowledgment: Signing the tender document constitutes acknowledgment that the bidder has reviewed and accepts all tender conditions. - Document Packaging: Each bid must be submitted in a separate, clearly labeled envelope with the project name clearly written on the exterior. ### Required Supporting Documents Bidders must attach the following documents with their bid submission: - Tax exemption certificate in the name of the Plan International Sudan White Nile Program Director - Legal stamp within the cost limit of the bid (to be provided after award to the successful bidder) - Zakat clearance certificate in the name of the organization director - Business registration/trade license - Valid Engineering Regulatory Council registration certificate (current) - Valid Contractors Union membership certificate (current) - Financial Solvency Certificate: Bank statement covering at least three (3) months - Experience Documentation: Certificates or evidence of completion of similar construction projects - Bid Bond: Bank check, insurance company guarantee, or bank guarantee letter in the amount of 2% of the tender value (in the name of the Plan International manager). This will be: - Forfeited if the bidder fails to honor the bid after award - Increased to 10% for successful bidders as a performance bond - Refunded to unsuccessful bidders - Document Submission Note: Submit copies of all required documents. Original documents will not be returned to bidders. ### Tender Document Distribution Tender brochures and detailed specifications are available from: Plan International Sudan – White Nile Program Office Location: Kosti, east of the Kosti Officers Club and south of the Sudanese Electricity Distribution Company Distribution Period: 20–28 September 2026 Official working hours only |  |
| --- | --- |

- The last day to receive tender brochures from companies and contractors is, Thursday: Thursday, 1 October 2026 at 10:00 AM

التاريخ: 20/09/2620م                                                                                Tender No. (WN 01 FY2027)

السلام عليكم ورحمة الله تعالى وبركاته

السادة /  ...................................................................

المحترم،،،،،،،

| Total USD | Pries USD |  | Quantities | Unit | المواصــــــــــــــــــــــــــــــــــــــفات |  | البند |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Job |  |  |  | نظافة وتجهيز وتسوية وتخطيط الموقع Cleaning, equipping, leveling and site planning |  | 1 |
| اعمــــــــــــــــــــــــــــــــــال الحفــــــــــــــــــــريات: |  |  |  |  |  |  | 2 |
|  |  |  | 56 | M.l | حفر وازالة ناتج الحفرمن الاساس الشريطي بعرض 50سم وعمق 90 سم للفصول Drilling and removal of the drilling output from the strip basis with a width of 50 cm and a depth of 90 cm for seasons |  | 2.1 |
| اعمـــــــــــــــــــــــــــــــــــــــال المبـــــــــــــــــــــاني: |  |  |  |  |  |  | 3 |
|  |  |  | 28 | M2 | توريد وتشييد قصة من الطوب الاحمر نمرة (1) والمونة الاسمنتية 1:6 ارتفاع 40سم حسب توجيهات المهندس المشرف Supplying and constructing a red brick story (1) and cement mortar 1: 6, height 40 cm, as directed by the supervising engineer |  | 3.1 |
|  |  |  | 168 | M2 | توريد وتشييد مباني من الطوب الاحمر نمرة (1)طوبة ونص والمونة الاسمنتية 1:6 لحوائط الفصول ارتفاع 3م Supply and construction of red brick buildings Nimra (1)brick and half and concrete mortar 1:6 for the walls of the seasons height 3 m |  | 3.2 |
|  |  |  | 28 | M2 | توريد وتشييد مباني بربيت من الطوب الاحمر واحد ونص طوبة والمونة الاسمنتية 1:6 ارتفاع 50سم Supply and construction of the buildings of The Barbit of the red brick one and a half brick and cement mortar 1:6 height 50 cm |  | 3.3 |
|  | Total building work |  |  |  |  |  |  |
| اعمـــــــــــــــــــــــــــــــــــال الردمــــــــــــــــيات The work of the back feeling s: : |  |  |  |  |  |  | 4 |
|  |  |  | 28 | م3 | توريد وتشييد ردمية من السفاية للاساس الشريطي سمك 50سم مع المندلة الجيدة والرش بالماء Supply and construction of earthworks from Sufaya for 50 cm striped foundation with good mandala and spraying with water |  | 4.1 |
|  |  |  | 22.4 | م3 | توريد وتشييد ردمية ترابية (حماري) سمك 40سم علي طبقتين مع المندلة الجيدة والرش بالماء لارضيات المباني Supply and construction of earthen plaster (donkey) 40 cm thickness on two floors with good mandala and water spray for building floors |  | 4.2 |
|  |  | total works of back feeling |  |  |  |  |  |
| اعمـــــــــــــــــــــــــــــــــــال الخرسانات : |  |  |  |  |  |  | 5 |
|  |  |  | 9 | م3 | توريد وتشييد فرشة بيضاء سمك 10سم بخلطة خرسانية 1:3:6لارضيات الفصول بفواصل تمدد مع المعالجة الجيدة Supply and construction of 10 cm white mattress with concrete mixture 1: 3: 6 for classroom flooring with expansion joints with good handling |  | 5.1 |
|  |  |  | 6.72 | م3 | توريد وتشييد وتوضيب قريد بيم (30*40) سم بحديد تسليح 4لينية (عدد6سيخات)ومسافات بينية 15سم ومونة خرسانية Supplying, constructing and packing P-Bum (30 * 40) cm with reinforcing steel 4 lenght (6 skewers), interstitial spacing of 15 cm and concrete mortar 1: 2: 4 1:2:4 |  | 5.2 |
|  |  |  | 3.5 | م3 | توريد وتشييد وتوضيب بيم عتب وهيد بيم (25*25) سم بحديد تسليح 4لينية(عدد4سيخات) ومسافات بينية 15سم ومونة خرسانية Supplying, constructing and packing beams of lintel (25 * 25) cm with reinforcing steel 4 lins (4 skewers), interfaces of 15 cm and concrete mortar 1: 2: 4 1:2:4 |  | 5.3 |
|  |  | جملة اعمال الخرسانات |  |  |  |  |  |
| اعمـــــــــــــــــــــــــــــــــــال التشطـــــــــــــــيبات : |  |  |  |  |  |  | 6 |
|  |  |  | 318 | M2 |  | توريد وتشييد بياض داخلي وخارجي سمك 2سم بالمونة الاسمنتية 1:8 لحوائط الفصول مع المعالجة الجيدة والرش بالماء وتشطيب الجلس والشوكات Supply and construction of indoor and outdoor laying thickness of 2 cm with 1: 8 cement mortar for classroom walls with good treatment, spraying with water, finishing the sat and thorns | 6.1 |
|  |  | Job |  |  |  | توريد وتشييد مسطبة للاستاذة بابعاد (3*2)م ومسطبة خارجية امام الفصل بعرض الباب وبتدرج مناسب Supply and construction of a terrace for the teacher with dimensions (3*2)M and an external terrace in front of the classroom by the width of the door and appropriately graded | 6.2 |
|  |  | Job |  |  |  | توريد وتشييد سبورة بابعاد (3*2.5)م كاملة مع الطلاء بالبوهية Supply and construction of a 3*2.5 m balboh paint | 6.3 |
|  |  |  | 318 | M2 |  | توريد وتشييد وعمل نقاشة ثلاثة اوجة بالطلية الحريرية والبوماستك من الداخل والخارج وطباعة شعار المنظمة والشركاء Supply, construction and work of a three-facet discussion in silk and pumastics from home and abroad and print the logo of the organization and partners | 6.4 |
|  |  | جملة اعمال التشطيبات |  |  |  |  |  |
| اعمــــــــــــــــــــــــــــــــــــــــال الحديد والسقوفات : |  |  |  |  |  |  | 7 |
|  |  |  | 2 | number |  | توريد وتوضيب وتركيب باب ضلفتين من المواسير المربعة اكومي (1.2*2.2) كامل Supply, packing and installation of two acome square pipes (1.2 * 2.2) complete | 7.1 |
|  |  |  | 10 | number |  | توريد وتوضيب وتركيب شباك من المواسير المربعة (1.2*2.2) كامل مع عمل الحماية Supplying, packing and installing square tubes (1.2 * 2.2) complete with protection work | 7.2 |
|  |  |  | 90 | M2 |  | توريد وتركيب وتثبيت سقف من الزنك العادي ابوحصان (35ملي) والكمر 14سم (عدد3) والمواسير المربعه التقيلة (4*8) للمدادات الطولية (عدد5)ومسمار الغانج وتثبيت عازل حراري بالنملي الناعم حسب اصول الصنعهSupply, installation and installation of the roof ordinary zinc abuhusan (35ml), and heavy square pipes (4*8) for longitudinal supplies (number5) and gang nail and install thermal insulation in soft (according to the origins of the manufacture) according to the origins of his workmanship | 7.3 |
|  |  | جملة اعمال الحـــــديد والسقوفات |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |

Summary

| Total Cost | Value Added | Price | Location | Description. |
| --- | --- | --- | --- | --- |
|  |  |  | Altonsa Community, Al-Dawaim Locality, Southern Rural | Construction of Classrooms (9m × 5m) |
|  | Total Cost |  |  |  |

الجملة كتابتاً ...........................................................................................

الفترة الزمنية للتسليم : .......................................................

قسم المشتروات:   محمد عبدالقادر محمود  التوقـــــيع ..................

لختم ..............

إسم الشركة \ المورد ................................... التوقيع ---------------

الختم...........................               التلفون......................................